Measurement System Analysis

Guideline

Revision History

| Version | Updated By | Date | Approved By | Change Request ID | Significant Changes |
| --- | --- | --- | --- | --- | --- |
| 1.0 | SEPG | 04-Jan-18 | Manoj Nagpal | NA | Initial Version |
| 1.1 | Prabhakar | 30-Mar-22 | Manish Singla | NA | Added the MSA frequency, highlighted in grey |

- Measurement System Analysis (MSA)

- Measurement system analysis (MSA) uses scientific tools to determine the amount of variation contributed by the measurement system. It is an objective method to assess the validity of a measurement system and minimize the factors contributing to process variation that is actual stemming from the measurement system.

- The purpose of measurement system analysis is to ensure and demonstrate that the measuring procedures and systems provide

    - Adequate resolution,

    - Results that are not unduly biased, and

    - Little variability in comparison with specified tolerances.

    - The frequency of MSA is “Yearly”

- Tool & Techniques used for MSA.

- Minitab Tool

- Hypothesis

- Control Chart

- MSA-details

- Measurement systems analysis assesses the adequacy of a measurement system for a given application. The MSA when done in depth, with the help of tools available, helps us in understanding the various components of variation. The variation in the measurement system and thereby providing the directions for the improvement of measurement system.

- Before you collect data from your process (for example, to analyze process control or capability). Or Change in the measure (inclusion of new measures, change in calculation formulae etc…) & Change in the team, that supports data collection.

- Use measurement system analysis to confirm that the measurement system measures consistently and accurately.

- The steps to conduct MSA:

- Select the measurement system to be analyzed.

- Take samples to be inspected.

- Select the people/operator who are doing the measurements on routine basis.

- Conduct MSA as per the type of data (Discrete or continuous).

    - Conduct statistical analysis with Minitab to analyze the variation and evaluate w.r.t success criteria.

    - Compare the results achieved through the statistical analysis. Identify the corrective actions for improving MSA, with planned dates for implementation and the responsible person.

- As per Data type, below method need to follow:-

- Continuous data- - The most commonly used technique is Gage R&R (Repeatability & Reproducibility)

- Measurement System characteristics

- Precision: The variation when the same part/unit is measured repeatedly with the same Instrument. Precision, or measurement variation has two components:-

- Repeatability: Variation in the measurement system. It is the variation that occurs when successive measurements are made in the same conditions including below:-

    - Same operator for data collection.

    - Same parts/unit being measured

    - Same Characteristic

    - Same Instrument of measurement.

    - Same set-up and environmental conditions

- Reproducibility: Variation in measurement when two or more persons/operator measure the same unit using the same instrument.

    - Different operator for data collection

    - Same parts/unit being measured

    - Same Characteristic

    - Same Instrument of measurement.

    - Same set-up and environmental conditions

- Threshold for acceptability  of the measurement system for continuous data type listed below:-

- R & R Less than 10% - Acceptable.

- R & R 10% to 30% - May be acceptable depending on conditions.(e.g.- cost of measuring/device, cost of repair, other factors)

- R & R over 30% - Not acceptable.

- Note: - For Gage R&R follow path from Minitab as below;

Stat > Quality Tools > Gage Study > Gage R&R Study (Crossed)

- Discrete data- Attribute Agreement Analysis (Fleiss' Kappa Method )

- Attribute Agreement Analysis is used to assess of agreement of nominal or ordinal rating given by multiple appraisers. Attribute Agreement Analysis is used to determine the accuracy and precision of the assessments made by appraisers and to identify which items have the highest misclassification rates.

- Kappa is indicates the degree of agreement of the nominal assessments made by          multiple appraisers when evaluating the same samples. Kappa values range from -1 to +1. The higher the value of kappa, the stronger the agreement, when:-

    - Kappa = 1, perfect agreement exists.

    - Kappa = 0, agreement is the same as would be expected by chance.

    - Kappa < 0, agreement is weaker than expected by chance; this rarely happens.

- Typically a kappa value of at least 0.70 is required, but kappa values close to 0.90 are preferred.

- When you have ordinal ratings, such as defect severity ratings on a scale of 1-5, Kendall's coefficients, which take ordering into consideration, are usually a more appropriate statistic to evaluate association than kappa alone.

- Note: - For Attribute Agreement Analysis follow path from Minitab as below.

Stat > Quality Tools > Attribute Agreement Analysis.